/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Main program body

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

#include <string.h>

/* Private variables ---------------------------------------------------------*/

UART_HandleTypeDef huart1;

osThreadId LedTaskHandle;

osThreadId ButtonTaskHandle;

osThreadId UartTaskHandle;

osMessageQId FreqQueueHandle;

/* 全局共享变量，用于串口读取 */

uint32_t g_freq = 500;

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_USART1_UART_Init(void);

void StartLedTask(void const * argument);

void StartButtonTask(void const * argument);

void StartUartTask(void const * argument);

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* MCU Configuration--------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_USART1_UART_Init();

/* Create the queue(s) */

osMessageQDef(FreqQueue, 16, uint32_t);

FreqQueueHandle = osMessageCreate(osMessageQ(FreqQueue), NULL);

/* Create the thread(s) */

osThreadDef(LedTask, StartLedTask, osPriorityNormal, 0, 128);

LedTaskHandle = osThreadCreate(osThread(LedTask), NULL);

osThreadDef(ButtonTask, StartButtonTask, osPriorityHigh, 0, 128);

ButtonTaskHandle = osThreadCreate(osThread(ButtonTask), NULL);

osThreadDef(UartTask, StartUartTask, osPriorityNormal, 0, 128);

UartTaskHandle = osThreadCreate(osThread(UartTask), NULL);

/* Start scheduler */

osKernelStart();

while (1)

{

}

}

/**

* @brief System Clock Configuration

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)

{

Error_Handler();

}

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief USART1 Initialization Function

*/

static void MX_USART1_UART_Init(void)

{

huart1.Instance = USART1;

huart1.Init.BaudRate = 115200;

huart1.Init.WordLength = UART_WORDLENGTH_8B;

huart1.Init.StopBits = UART_STOPBITS_1;

huart1.Init.Parity = UART_PARITY_NONE;

huart1.Init.Mode = UART_MODE_TX_RX;

huart1.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart1.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart1) != HAL_OK)

{

Error_Handler();

}

}

/**

* @brief GPIO Initialization Function

*/

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

/*Configure GPIO pin Output Level */

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_13, GPIO_PIN_RESET);

/*Configure GPIO pin : PC13 (LED) */

GPIO_InitStruct.Pin = GPIO_PIN_13;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/*Configure GPIO pin : PA0 (Button) */

GPIO_InitStruct.Pin = GPIO_PIN_0;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

}

/* ============================================================================ */

/* ======================== 任务核心逻辑区 ===================================== */

/* ============================================================================ */

/* 1. LED 任务：接收队列消息，控制闪烁 */

void StartLedTask(void const * argument)

{

uint32_t delay_time = 500;

for(;;)

{

/* 尝试从队列获取新频率，最多等待 delay_time 毫秒 */

osEvent evt = osMessageGet(FreqQueueHandle, delay_time);

/* 如果等到了新消息，更新闪烁周期 */

if(evt.status == osEventMessage) {

delay_time = evt.value.v;

}

/* 翻转LED电平 */

HAL_GPIO_TogglePin(GPIOC, GPIO_PIN_13);

}

}

/* 2. 按键任务：扫描按键，发送队列，更新全局变量 */

void StartButtonTask(void const * argument)

{

for(;;)

{

/* 模拟按键按下（PA0接地） */

if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0) == GPIO_PIN_RESET) {

osDelay(20); /* 消抖 */

if (HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0) == GPIO_PIN_RESET) {

/* 临界区保护：安全地修改全局频率变量 */

taskENTER_CRITICAL();

if (g_freq == 200) g_freq = 500;

else if (g_freq == 500) g_freq = 1000;

else g_freq = 200;

uint32_t send_freq = g_freq; /* 暂存准备发送的值 */

taskEXIT_CRITICAL();

/* 将新频率放进队列，通知 LED 任务 */

osMessagePut(FreqQueueHandle, send_freq, 10);

/* 死循环等待按键松开（线拿开），防止按一次疯狂触发 */

while(HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_0) == GPIO_PIN_RESET) {

osDelay(20);

}

}

}

osDelay(20); /* 按键任务每 20ms 扫描一次 */

}

}

/* 3. 串口任务：读取全局变量，每秒打印 */

void StartUartTask(void const * argument)

{

char msg[50];

for(;;)

{

/* 临界区保护：安全地读取全局频率变量 */

taskENTER_CRITICAL();

uint32_t current_f = g_freq;

taskEXIT_CRITICAL();

/* 组装并发送字符串 */

int len = sprintf(msg, "Current LED Freq: %lu ms\r\n", current_f);

HAL_UART_Transmit(&huart1, (uint8_t*)msg, len, 100);

/* 串口每 1s 输出一次信息 */

osDelay(1000);

}

}

/**

* @brief  This function is executed in case of error occurrence.

*/

void Error_Handler(void)

{

__disable_irq();

while (1)

{

}

}